岩石样品Pb同位素组成分析流程

1、称取适量样品置于聚四氟乙烯密封溶样罐，硅酸岩样品加入2mL氢氟酸和1mL 硝酸，180℃条件下密闭溶解样品，开盖蒸干后另外加入1mL盐酸和1mL硝酸，再次于180℃条件下溶样48h后再蒸干。待样品蒸干后，加入6mol/l 盐酸溶解，再次蒸干（重复操作1次）。加入适量HBr（1mol/L）和HCl（2mol/L）的混合酸。离心，将上层清液加入AG-1×8阴离子树脂柱，依次用0.3mol/L氢溴酸（HBr）和0.5mol/L盐酸（HCl）淋洗杂质。最后用7mL 6mol/L盐酸解吸铅，蒸干后加入0.5mL 6mol/L硝酸并蒸干。加入4mL 2%硝酸溶液，并加入10μL 10μg/g Tl标准溶液作为内标。

2、Pb同位素组成分析在多接收电感耦合等离子体质谱仪Neptune Plus上完成，用203Tl/205Tl比值（0.418652）对测量的铅同位素比值进行质量分馏校正；分析过程中采用标准物质NBS981监控质谱仪工作状态，207Pb/206Pb、208Pb/204Pb、207Pb/204Pb和206Pb/204Pb 平均值与其推荐值在误差范围内一致，表明测试结果可信可靠。使用元素含量标样BCR-2作为监控样监控测试流程，其测试结果与BCR-2多家实验室测试数据（表1）在误差范围内一致。

3、Pb的全流程空白为n×10-9g。

详细的Pb同位素分析流程详见：

卢山松, 童喜润, 谭娟娟, 邱啸飞, 杨红梅. 高精度铅同位素比值MC-ICP-MS测试方法及岩石标样铅同位素组成. 地质通报, 2023, 42(9): 1516-1530.

表1监控样品BCR-2的文献数据平均值

| 来源 | 206Pb/204Pb | 2σ | 207Pb/204Pb | 2σ | 208Pb/204Pb | 2σ |
| --- | --- | --- | --- | --- | --- | --- |
| 文献数据平均值 | 18.757 | 0.009 | 15.624 | 0.006 | 38.725 | 0.025 |

注：文献数据平均值为统计了7篇文献报道值后计算出的平均值。7篇文献包括：

1）Woodhead Jon D, Hergt M. 2000. Pb-isotope analyses of USGS reference materials. Geostandards and Geoanalytical Research. 24(1):33-38.

2）Collerson Kenneth D, Kamber Balz S, Schoenberg Ronny. 2002. Applications of accurate, high-precision Pb isotope ratio measurement by multi-collector ICP-MS. Chemical Geology. 188:65-83.

3）Li Huaikun, Niu Yaoling. 2003. Multi-collector ICP-MS analysis of Pb isotope ratios in rocks:Data, Procedure and Caution. ACTA GEOLOGICA SINICA. 77(1):44-58.

4）Baker J, Peata D, Waight T, Meyzen C. 2004. Pb isotopic analysis of standards and samples using a 207Pb–204Pb double spike and thallium to correct for mass bias with a double-focusing MC-ICP-MS. Chemical Geology, 211:275-303.

5）Weis Dominique, Kieffer Bruno, Maerschalk Claude, et al. 2006. High-precision isotopic characterization of USGS reference materials by TIMS and MC-ICP-MS. Geochemistry Geophysics Geosystems. 7(8).

6）刘希军, 王桂琴, Castillo P R, 等. 2013. 高精度铅同位素207Pb-204Pb双稀释剂TIMS测试方法. 地球化学, 42(2):103-115.

7）李世珍，马健雄，朱祥坤，等. 2015. 离子交换分离过程中铅同位素分馏评估及针对MC-ICPMS铅同位素测定的分离纯化方法的修正. 岩石矿物学杂志, 35(5):785-792.